Programmable Getting Started

內容

1	安裝開發環境	1

2	編譯Example	2

3	上傳檔案到Device	4

4	Add new project	9

# 安裝開發環境

  - Tool Chain: Sourcery CodeBench Lite ARM EABI Release

    - http://www.mentor.com/embedded-software/sourcery-tools/sourcery-codebench/editions/lite-edition/

    - 安裝後修改環境變數”PATH”使之可以在任何資料夾執行

      - 電腦右鍵內容進階系統設定進階環境變數PATH中增加路徑…Sourcery_CodeBench_Lite_for_ARM_EABI\bin

      - 重啟電腦使之生效

    - 驗證

      - MS-DOS下執行arm-none-eabi-gcc

  - IDE: eclipse

    - http://www.eclipse.org/downloads/

    - 確認有安裝C/C++的部分

    - you will need to install some Java virtual machine (JVM). You may install a Java Runtime Environment (JRE). http://www.oracle.com/technetwork/java/javase/downloads/index.html

# 編譯Example

  - 執行eclipse, select a workspace: 選擇BSDrls\Examples的路徑

  - Workbench畫面，無需任何設定

    - 檢查是否在C/C++環境(右上角)

    - Makefile均已寫好，再檢查一下。

      - 右邊Make TargetEdit

    - Build:不使用eclipse的設定，而是直接執行Makefile

    - Build完畢，在E01_echoServer_netconn_api\build\gcc\output生成執行檔案user_main.aff

# 上傳檔案到Device

  - Hardware setup

    - Please use switch to connect the LAN of computer and device. Do not connect computer and device directly.

  - Install Manager Utility

  - Start Manager Utility

    - Change the properties of the Manager Utility

    - Click right-hand buttonpropertiescheck “Run this program in compatibility mode for” windows XP, and “Run this program as an administrator” Apply

    - Allow access both “Private” and “Public”. It is important for upgrade.

  - Main

    - Search&configure tool

  - Double click the device

  - And then enter configure device

  - 右邊Upgrade按鈕,選擇剛編譯過的user_main.aff, 按OK

  - 確認版本號，FMW為kernel版本

  - 更新完畢重啟

  - 串口Console

    - 115200, N81, ANSI(Terminal type)

    - 開機訊息/fail訊息都會顯示在這裡

    - 開發過程中的debug message也可以透過console_write印在這裡

# Add new project

  - Add new project under /Programmable212

  - Click right-hand button on eclipseProject ExploerPorgrammable212

  - New folder appeared

  - And then repeat 2.2.2.1 to build the project